Top of Form

|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| DUST MOP/DUST CLOTH TREATMENT (BULK) Product Number: 3013; 3213 | Janitorial |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Cleaning compounds, liquid, n.o.s. |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 0 - Minimal | Health: | 0 - Minimal |
| Fire: | 0 - Minimal | Fire: | 0 - Minimal |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | A - Safety Glasses, gloves |

SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| Mineral Seal Oil | 01-05 | 64742-46-7 | 5 [mineral oil mist] | Not Established | Not Established | No |
| *Ethylene glycol | 3 | 107-21-1 | Not Established | Not Established | 127 (ACGIH) | No |

Comment: *This substance is subject to the reporting requirements of Section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 (SARA) and of 40 CFR 372.

SECTION III: PHYSICAL DATA

| Boiling Point: | 210-212 °F | Vapor Pressure: | 18 mm Hg @ 75°F |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Forms an emulsion |
| pH: | 8.5-9.0 | Specific Gravity (H2O=1): | 1.0 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | 10-12 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Opaque white, non-viscous liquid, fresh scent |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | None | Method Used: | ASTM-D56 |
| --- | --- | --- | --- |
| Flammable Limits: | N/A | Flame Extension: | N/A |

| Extinguishing Media: | N/A |
| --- | --- |
| Special Fire Fighting Procedures: | N/A |
| Unusual Fire & Explosive Hazards: | N/A |

SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not established | Primary Routes of Entry: |  |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | May cause eye and skin irritation. Harmful if swallowed. Prolonged breathing of vapor or mist may cause respiratory irritation. Avoid contact with eyes and skin. Do not swallow. Avoid prolonged breathing of spray mist. Prolonged exposure to ethylene glycol may cause adverse effects to reproductive system. |
| --- | --- |
| Conditions Aggravated by Use: | Unknown |

Emergency & First Aid Procedures:

| Eyes: | In case of contact; immediately flush eyes with water for at least 15 minutes. Remove contact lenses. If irritation persists; get medical attention. |
| --- | --- |
| Skin: | Wash with soap and water. If irritation persists; get medical attention. |
| Ingestion: | If swallowed; do not induce vomiting. drink large quantities of water. Get medical attention immediately. Never give anything by mouth to an unconscious person. |
| Inhalation: | In case of respiratory irritation; move person to fresh air. |

SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | Strong oxidizers |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | CO, CO2 | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | For small spills, clean up with absorbent material and incinerate. Large spills should be contained and disposed of in accordance with local, state or Federal regulations |
| --- | --- |
| Waste Disposal Method: | Same as above |

SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | None required |
| --- | --- |
| Ventilation: | Good general ventilation should be sufficient for most conditions. Local exhaust ventilation may be necessary for some operations. |
| Protective Gloves(Specify Type): | Not required |
| Eye Protection(Specify Type): | Goggles |
| Other Protective Equipment: | None necessary |

SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Keep from freezing |
| --- | --- |
| Other Precautions: | None |

| © SCC 09/23/99 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| DUST MOP/DUST CLOTH TREATMENT (BULK) | Effective Date: | 09/23/99 | Supercedes: | 02/06/98 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | Update Section II & V, Address and Format |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.

Bottom of Form